# Functionally graded roofs for passive snow management: a reduced-order multiobjective modeling study

Tatsuki Onishi*

REQUIRED

Corresponding author: Tatsuki Onishi, REQUIRED, bougtoir@gmail.com

# Abstract

Passive roof-snow strategies face a modeled trade-off between retaining snow mass and releasing large discrete snow-shedding events. This study tests whether spatial variation in roof slope and snow-surface interaction changes that trade-off relative to optimized spatially uniform roofs. A unit-width roof was discretized into cells with slope, static and kinetic friction, adhesion, snow mass, age, volume, and density states. Synthetic weather scenarios were used for primary optimization; official Japanese Meteorological Agency daily observations provided supplementary scenario evaluation rather than validation. Uniform designs were mapped before checkpointed, multi-seed NSGA-II optimization of geometry-only, surface-only, and joint heterogeneous profiles. Primary outcomes were maximum modeled roof snow mass per metre and maximum eave-release mass per timestep. Convergence, ablation, parameter sensitivity, Monte Carlo perturbation, continuous-to-discrete surface mapping, and labor-scarcity decision weights were evaluated. At the normalized-distance knee, the selected joint profile had Lmax=3210.6 kg m-1 and Smax=40.00 kg m-1. Relative to the selected uniform knee, these values changed by +3913.3% and -50.0%, respectively. This knee comparison is illustrative and does not establish front-wide superiority. The results define conditional modeled trade-offs, not structural or pedestrian safety. The reduced-order model omits wind redistribution and requires calibration against roof-scale observations before design use.

Keywords: roof snow; passive snow management; multiobjective optimization; spatial heterogeneity; snow shedding; reduced-order model

# 1. Introduction

Roof snow is spatially variable, evolves with weather, and can either remain on the roof or leave in discrete releases. Roof-scale experiments and deposition studies show that geometry, wind, and scale influence snow distributions, while validation remains essential for computational models [1-6]. Adjacent photovoltaic, membrane-roof, meteorological, and field studies further motivate explicit sensitivity analysis and independent validation [7-12].

Japanese technical and administrative sources distinguish retention, shedding, melting, and load-resisting strategies and document the importance of surface condition, aging, snow guards, drainage, cornices, and site context [13-21]. These sources do not establish universal material coefficients or safety thresholds. They instead motivate the present comparison of generic spatially uniform and graded design spaces.

The research question was frozen before production analysis: under identical modeled environmental conditions, how does allowing spatial variation in roof geometry and snow-surface interaction change the achievable trade-off between retained modeled snow mass and discrete shedding events compared with optimized uniform roofs? The hypothesis that heterogeneity expands the attainable design space was tested without assuming superiority.

# 2. Methods

## 2.1 Reduced-order roof-snow model

An 8 m long, 1 m wide roof was represented by 24 ridge-to-eave cells. Each cell carried slope, static friction, kinetic friction, adhesion, snow mass, mass-weighted age, volume, and density. Snowfall was added before each within-step maximum was recorded. Melt removed mass, volume, and age moment proportionally. Compaction changed volume and diagnostic depth. Density converted observed snowfall depth to mass but was not asserted to change basal sliding at fixed mass.

A cell became mobile when downslope gravity exceeded static friction plus area-scaled adhesion. Kinetic friction controlled transport after release. Mass, age moment, and volume moved together. Snow leaving the final cell was an eave-shedding event. The kinetic-energy metric was a roof-wide specific-energy proxy, not an impact or injury model.

F_g = m g sin(theta)

F_r = mu_s m g cos(theta) + A tau_a

## 2.2 Outcomes and comparison sets

The locked primary outcomes were Lmax, the maximum modeled roof snow mass per metre width immediately after snowfall and before same-step melt or release, and Smax, the maximum eave-release mass in one timestep per metre width. Secondary outcomes included total shed mass, event count, event-mass summaries, Snow Shedding Concentration Index (SSCI), kinetic-energy proxy, intervention-threshold time, manual triggers, residual mass, maximum modeled snow depth, total variation, transition count, and discretization loss.

SSCI = sum_j (m_j / M_total)^2

Comparators were retention-oriented, shedding-oriented, and intermediate uniform baselines; the complete optimized uniform frontier; and geometry-only, surface-only, and joint heterogeneous frontiers. Figure 1 summarizes the model and optimization workflow; Figure 2 summarizes synthetic and observed weather contexts; Figures 3-5 show the primary frontier, selected profile, and ablation frontiers. Tables 1-3 report frontier metrics, the selected profile, and continuous-to-discrete mapping.

Figure 1. Reduced-order model and comparative optimization workflow.

Figure 2. Synthetic snowfall scenarios and JMA ground-snow contexts.

Figure 3. Primary modeled retained-mass and release-mass Pareto comparison.

Figure 4. Selected graded joint profile and generic mapped surface classes.

Figure 5. Geometry-only, surface-only, and joint heterogeneous Pareto fronts.

### Table 1. Pareto-front summary.

| Class | Pareto n | Knee Lmax | Knee Smax | Norm. HV | Epsilon vs uniform | Dominated fraction |
| --- | --- | --- | --- | --- | --- | --- |
| uniform | 2912 | 80.00 | 80.00 | 0.402 | 0.000 | 0.818 |
| geometry | 320 | 175.3 | 105.1 | 0.267 | 0.205 | 0.978 |
| surface | 1731 | 80.00 | 80.00 | 0.402 | 0.000 | 0.030 |
| joint | 1928 | 3210.6 | 40.00 | 0.611 | 0.000 | 0.009 |

### Table 2. Selected joint-profile cell properties.

| Cell | Continuous slope | Discrete slope | Continuous friction | Continuous adhesion | Generic class |
| --- | --- | --- | --- | --- | --- |
| 1 | 39.97 | 33.00 | 0.464 | 202.3 | rough_retention |
| 2 | 35.51 | 33.00 | 0.474 | 176.3 | rough_retention |
| 3 | 31.04 | 33.00 | 0.484 | 150.3 | rough_retention |
| 4 | 26.58 | 33.00 | 0.494 | 124.3 | rough_retention |
| 5 | 22.11 | 21.00 | 0.504 | 98.30 | aged_coated_metal |
| 6 | 17.64 | 21.00 | 0.514 | 72.31 | aged_coated_metal |
| 7 | 13.18 | 21.00 | 0.524 | 46.32 | aged_coated_metal |
| 8 | 8.710 | 21.00 | 0.534 | 20.33 | aged_coated_metal |
| 9 | 6.863 | 12.00 | 0.526 | 2.890 | aged_coated_metal |
| 10 | 10.26 | 12.00 | 0.482 | 2.557 | aged_coated_metal |
| 11 | 13.65 | 12.00 | 0.439 | 2.224 | aged_coated_metal |
| 12 | 17.05 | 12.00 | 0.395 | 1.891 | aged_coated_metal |
| 13 | 20.44 | 24.00 | 0.352 | 1.558 | smooth_metal |
| 14 | 23.84 | 24.00 | 0.308 | 1.225 | smooth_metal |
| 15 | 27.23 | 24.00 | 0.265 | 0.893 | smooth_metal |
| 16 | 30.62 | 33.00 | 0.221 | 0.560 | smooth_metal |
| 17 | 32.84 | 33.00 | 0.221 | 1.181 | smooth_metal |
| 18 | 34.48 | 33.00 | 0.241 | 2.279 | smooth_metal |
| 19 | 36.11 | 38.00 | 0.262 | 3.377 | smooth_metal |
| 20 | 37.74 | 38.00 | 0.283 | 4.475 | smooth_metal |
| 21 | 39.37 | 38.00 | 0.304 | 5.573 | smooth_metal |
| 22 | 41.01 | 43.00 | 0.324 | 6.671 | aged_coated_metal |
| 23 | 42.64 | 43.00 | 0.345 | 7.768 | aged_coated_metal |
| 24 | 44.27 | 43.00 | 0.366 | 8.866 | aged_coated_metal |

### Table 3. Continuous-to-discrete profile loss.

| Mapping | Lmax | Smax | Mean SSCI | Event count | Slope transitions | Surface transitions | Lmax loss | Smax loss |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| continuous | 3210.6 | 40.00 | 0.013 | 137.7 | 23 | 23 | 0.000 | 0.000 |
| discrete_generic | 3222.3 | 44.41 | 0.018 | 74.33 | 6 | 3 | 11.68 | 4.412 |

## 2.3 Weather forcing

Primary optimization used 3 deterministic synthetic winters spanning cold persistent, maritime warming, and severe inland conditions, including repeated snowfall and configured rain-on-snow periods. JMA daily mean temperature, precipitation, snowfall depth, and ground snow depth were downloaded as immutable official HTML snapshots for 4 stations and 3 winters. Original value strings and parsed numeric fields were retained. JMA ground observations were used only as scenario forcing and were not treated as direct roof-load measurements.

## 2.4 Optimization and analysis

A full 30 x 30 x 7 uniform slope-friction-adhesion map preceded heterogeneous optimization. Geometry-only, surface-only, and joint profiles were parameterized by 4 control points interpolated across the roof. Checkpointed NSGA-II used a population of 48, 45 generations, and seeds 1701, 2903, 4517. Feasible historical evaluations from every seed were combined and filtered to complete nondominated sets. Normalized hypervolume, additive epsilon versus the uniform front, combined-set dominance, and matched-objective improvement were calculated. Per-seed hypervolume and additive epsilon summarized optimizer stochasticity in a machine-readable supplement. Figure 6 reports numerical convergence; Figure 7 reports prespecified one-at-a-time sensitivity.

Figure 6. Numerical convergence relative to the finest tested resolution.

Figure 7. Prespecified one-at-a-time sensitivity of primary outcomes.

Monte Carlo robustness used 250 deterministic draws perturbing friction, adhesion, fresh-snow density, snowfall, and temperature for prespecified candidates sampled across the uniform and joint fronts. Continuous joint profiles were mapped to 3 generic surface classes and piecewise roof segments of at least 3 cells, with at most 6 transitions, without claiming proprietary material performance. Frozen manual-intervention weights generated low, medium, and high labor-scarcity choices across configured snowfall, shedding-penalty, and allowable-release scenarios. Figures 8-9 and Tables 4-8 summarize these analyses.

Figure 8. Monte Carlo perturbation distributions for frontier candidates.

Figure 9. Labor-scarcity selections and frozen strategy classifications.

### Table 4. Strategy phase-diagram classifications (selected columns; the complete table is supplied as CSV).

| Snowfall factor | Shed penalty | Labor scarcity | Allowable shed | Feasible | Class | Strategy | Manual triggers |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0.500 | 0.250 | low | 20.00 | True | joint | GRADED_HYBRID | 0.667 |
| 0.500 | 0.250 | low | 80.00 | True | uniform | SHEDDING | 0.000 |
| 0.500 | 0.250 | low | 200.0 | True | uniform | SHEDDING | 0.000 |
| 0.500 | 0.250 | medium | 20.00 | True | joint | GRADED_HYBRID | 0.667 |
| 0.500 | 0.250 | medium | 80.00 | True | uniform | SHEDDING | 0.000 |
| 0.500 | 0.250 | medium | 200.0 | True | uniform | SHEDDING | 0.000 |
| 0.500 | 0.250 | high | 20.00 | True | joint | GRADED_HYBRID | 0.667 |
| 0.500 | 0.250 | high | 80.00 | True | uniform | SHEDDING | 0.000 |
| 0.500 | 0.250 | high | 200.0 | True | uniform | SHEDDING | 0.000 |
| 0.500 | 1.000 | low | 20.00 | True | joint | GRADED_HYBRID | 0.667 |
| 0.500 | 1.000 | low | 80.00 | True | uniform | SHEDDING | 0.000 |
| 0.500 | 1.000 | low | 200.0 | True | uniform | SHEDDING | 0.000 |
| 0.500 | 1.000 | medium | 20.00 | True | joint | GRADED_HYBRID | 0.667 |
| 0.500 | 1.000 | medium | 80.00 | True | uniform | SHEDDING | 0.000 |
| 0.500 | 1.000 | medium | 200.0 | True | uniform | SHEDDING | 0.000 |
| 0.500 | 1.000 | high | 20.00 | True | joint | GRADED_HYBRID | 0.667 |
| 0.500 | 1.000 | high | 80.00 | True | uniform | SHEDDING | 0.000 |
| 0.500 | 1.000 | high | 200.0 | True | uniform | SHEDDING | 0.000 |
| 0.500 | 4.000 | low | 20.00 | True | joint | RETENTION | 1.000 |
| 0.500 | 4.000 | low | 80.00 | True | joint | RETENTION | 1.000 |
| 0.500 | 4.000 | low | 200.0 | True | joint | RETENTION | 1.000 |
| 0.500 | 4.000 | medium | 20.00 | True | joint | GRADED_HYBRID | 0.667 |
| 0.500 | 4.000 | medium | 80.00 | True | uniform | SHEDDING | 0.000 |
| 0.500 | 4.000 | medium | 200.0 | True | uniform | SHEDDING | 0.000 |
| 0.500 | 4.000 | high | 20.00 | True | joint | GRADED_HYBRID | 0.667 |
| 0.500 | 4.000 | high | 80.00 | True | uniform | SHEDDING | 0.000 |
| 0.500 | 4.000 | high | 200.0 | True | uniform | SHEDDING | 0.000 |
| 1.000 | 0.250 | low | 20.00 | True | joint | GRADED_HYBRID | 1.000 |
| 1.000 | 0.250 | low | 80.00 | True | uniform | SHEDDING | 0.000 |
| 1.000 | 0.250 | low | 200.0 | True | uniform | SHEDDING | 0.000 |
| 1.000 | 0.250 | medium | 20.00 | True | joint | GRADED_HYBRID | 1.000 |
| 1.000 | 0.250 | medium | 80.00 | True | uniform | SHEDDING | 0.000 |
| 1.000 | 0.250 | medium | 200.0 | True | uniform | SHEDDING | 0.000 |
| 1.000 | 0.250 | high | 20.00 | True | joint | GRADED_HYBRID | 1.000 |
| 1.000 | 0.250 | high | 80.00 | True | uniform | SHEDDING | 0.000 |
| 1.000 | 0.250 | high | 200.0 | True | uniform | SHEDDING | 0.000 |
| 1.000 | 1.000 | low | 20.00 | True | joint | GRADED_HYBRID | 1.000 |
| 1.000 | 1.000 | low | 80.00 | True | uniform | SHEDDING | 0.000 |
| 1.000 | 1.000 | low | 200.0 | True | uniform | SHEDDING | 0.000 |
| 1.000 | 1.000 | medium | 20.00 | True | joint | GRADED_HYBRID | 1.000 |
| 1.000 | 1.000 | medium | 80.00 | True | uniform | SHEDDING | 0.000 |
| 1.000 | 1.000 | medium | 200.0 | True | uniform | SHEDDING | 0.000 |
| 1.000 | 1.000 | high | 20.00 | True | joint | GRADED_HYBRID | 1.000 |
| 1.000 | 1.000 | high | 80.00 | True | uniform | SHEDDING | 0.000 |
| 1.000 | 1.000 | high | 200.0 | True | uniform | SHEDDING | 0.000 |
| 1.000 | 4.000 | low | 20.00 | True | joint | RETENTION | 1.000 |
| 1.000 | 4.000 | low | 80.00 | True | joint | RETENTION | 1.000 |
| 1.000 | 4.000 | low | 200.0 | True | joint | RETENTION | 1.000 |
| 1.000 | 4.000 | medium | 20.00 | True | joint | RETENTION | 1.000 |
| 1.000 | 4.000 | medium | 80.00 | True | joint | RETENTION | 1.000 |
| 1.000 | 4.000 | medium | 200.0 | True | joint | RETENTION | 1.000 |
| 1.000 | 4.000 | high | 20.00 | True | joint | RETENTION | 1.000 |
| 1.000 | 4.000 | high | 80.00 | True | uniform | SHEDDING | 0.000 |
| 1.000 | 4.000 | high | 200.0 | True | uniform | SHEDDING | 0.000 |
| 1.500 | 0.250 | low | 20.00 | True | joint | GRADED_HYBRID | 1.000 |
| 1.500 | 0.250 | low | 80.00 | True | joint | GRADED_HYBRID | 1.000 |
| 1.500 | 0.250 | low | 200.0 | True | uniform | SHEDDING | 0.000 |
| 1.500 | 0.250 | medium | 20.00 | True | joint | GRADED_HYBRID | 1.000 |
| 1.500 | 0.250 | medium | 80.00 | True | joint | GRADED_HYBRID | 1.000 |
| 1.500 | 0.250 | medium | 200.0 | True | uniform | SHEDDING | 0.000 |
| 1.500 | 0.250 | high | 20.00 | True | joint | GRADED_HYBRID | 1.000 |
| 1.500 | 0.250 | high | 80.00 | True | joint | GRADED_HYBRID | 1.000 |
| 1.500 | 0.250 | high | 200.0 | True | uniform | SHEDDING | 0.000 |
| 1.500 | 1.000 | low | 20.00 | True | joint | GRADED_HYBRID | 1.000 |
| 1.500 | 1.000 | low | 80.00 | True | joint | GRADED_HYBRID | 1.000 |
| 1.500 | 1.000 | low | 200.0 | True | uniform | SHEDDING | 0.000 |
| 1.500 | 1.000 | medium | 20.00 | True | joint | GRADED_HYBRID | 1.000 |
| 1.500 | 1.000 | medium | 80.00 | True | joint | GRADED_HYBRID | 1.000 |
| 1.500 | 1.000 | medium | 200.0 | True | uniform | SHEDDING | 0.000 |
| 1.500 | 1.000 | high | 20.00 | True | joint | GRADED_HYBRID | 1.000 |
| 1.500 | 1.000 | high | 80.00 | True | joint | GRADED_HYBRID | 1.000 |
| 1.500 | 1.000 | high | 200.0 | True | uniform | SHEDDING | 0.000 |
| 1.500 | 4.000 | low | 20.00 | True | joint | GRADED_HYBRID | 1.000 |
| 1.500 | 4.000 | low | 80.00 | True | joint | GRADED_HYBRID | 1.000 |
| 1.500 | 4.000 | low | 200.0 | True | uniform | SHEDDING | 0.000 |
| 1.500 | 4.000 | medium | 20.00 | True | joint | GRADED_HYBRID | 1.000 |
| 1.500 | 4.000 | medium | 80.00 | True | joint | GRADED_HYBRID | 1.000 |
| 1.500 | 4.000 | medium | 200.0 | True | uniform | SHEDDING | 0.000 |
| 1.500 | 4.000 | high | 20.00 | True | joint | GRADED_HYBRID | 1.000 |
| 1.500 | 4.000 | high | 80.00 | True | joint | GRADED_HYBRID | 1.000 |
| 1.500 | 4.000 | high | 200.0 | True | uniform | SHEDDING | 0.000 |

### Table 5. Frozen labor-scarcity decision choices.

| Labor scarcity | Design class | Strategy | Lmax | Smax | Manual triggers |
| --- | --- | --- | --- | --- | --- |
| low | uniform | SHEDDING | 80.00 | 80.00 | 0.000 |
| medium | uniform | SHEDDING | 80.00 | 80.00 | 0.000 |
| high | uniform | SHEDDING | 80.00 | 80.00 | 0.000 |

### Table 6. Complexity metrics for selected frontier rows.

| Class | Lmax | Smax | Max slope change | Transitions | Min segment cells | Min segment met | Transition limit met |
| --- | --- | --- | --- | --- | --- | --- | --- |
| uniform | 80.00 | 80.00 | 0.000 | 0 | 24 | True | True |
| uniform | 6417.0 | 0.000 | 0.000 | 0 | 24 | True | True |
| uniform | 6417.0 | 0.000 | 0.000 | 0 | 24 | True | True |
| geometry | 159.7 | 122.4 | 2.682 | 23 | 1 | False | False |
| geometry | 170.5 | 111.6 | 1.583 | 15 | 1 | False | False |
| geometry | 6417.0 | 0.000 | 4.944 | 17 | 1 | False | False |
| surface | 80.00 | 80.00 | 0.000 | 1 | 11 | True | True |
| surface | 80.00 | 80.00 | 0.000 | 1 | 5 | True | True |
| surface | 6417.0 | 0.000 | 0.000 | 6 | 1 | False | True |
| joint | 80.00 | 80.00 | 1.012 | 15 | 1 | False | False |
| joint | 80.00 | 80.00 | 1.243 | 16 | 1 | False | False |
| joint | 6417.0 | 0.000 | 1.794 | 17 | 1 | False | False |

### Table 7. Complexity-penalty selections.

| Penalty | Class | Lmax | Smax | Transitions | Score |
| --- | --- | --- | --- | --- | --- |
| 0.000 | uniform | 80.00 | 80.00 | 0 | 0.654 |
| 0.100 | uniform | 80.00 | 80.00 | 0 | 0.654 |
| 0.250 | uniform | 80.00 | 80.00 | 0 | 0.654 |

### Table 8. Monte Carlo robustness summary for candidate designs.

| Candidate | Class | Nominal Lmax | Nominal Smax | Q95 Lmax | Q95 Smax | P(manual) | Robust selected |
| --- | --- | --- | --- | --- | --- | --- | --- |
| joint_00 | joint | 80.00 | 80.00 | 3010.1 | 1012.6 | 0.116 | False |
| joint_01 | joint | 80.00 | 80.00 | 2909.3 | 1013.8 | 0.112 | False |
| joint_02 | joint | 80.00 | 80.00 | 103.8 | 103.8 | 0.000 | True |
| joint_03 | joint | 80.00 | 80.00 | 104.4 | 103.8 | 0.000 | False |
| joint_04 | joint | 80.00 | 80.00 | 103.8 | 103.8 | 0.000 | False |
| joint_05 | joint | 80.00 | 80.00 | 103.8 | 103.8 | 0.000 | False |
| joint_06 | joint | 80.00 | 80.00 | 110.1 | 110.1 | 0.000 | False |
| joint_07 | joint | 80.00 | 80.00 | 627.2 | 510.1 | 0.040 | False |
| joint_08 | joint | 80.00 | 80.00 | 114.4 | 111.9 | 0.000 | False |
| joint_09 | joint | 80.00 | 80.00 | 103.8 | 103.8 | 0.000 | False |
| joint_10 | joint | 80.00 | 80.00 | 103.8 | 103.8 | 0.000 | False |
| joint_11 | joint | 3210.6 | 40.00 | 4200.4 | 226.0 | 1.000 | False |
| joint_12 | joint | 6417.0 | 0.000 | 8324.5 | 0.000 | 1.000 | False |
| uniform_00 | uniform | 80.00 | 80.00 | 7465.5 | 5482.6 | 0.432 | False |
| uniform_01 | uniform | 80.00 | 80.00 | 103.8 | 103.8 | 0.000 | False |
| uniform_02 | uniform | 80.00 | 80.00 | 2211.4 | 655.8 | 0.068 | False |
| uniform_03 | uniform | 6417.0 | 0.000 | 8324.5 | 0.000 | 1.000 | False |
| uniform_04 | uniform | 6417.0 | 0.000 | 8324.5 | 0.000 | 1.000 | False |
| uniform_05 | uniform | 6417.0 | 0.000 | 8324.5 | 0.000 | 1.000 | False |
| uniform_06 | uniform | 6417.0 | 0.000 | 8324.5 | 0.000 | 1.000 | False |
| uniform_07 | uniform | 6417.0 | 0.000 | 8324.5 | 0.000 | 1.000 | False |
| uniform_08 | uniform | 6417.0 | 0.000 | 8324.5 | 0.000 | 1.000 | False |
| uniform_09 | uniform | 6417.0 | 0.000 | 8324.5 | 0.000 | 1.000 | False |
| uniform_10 | uniform | 6417.0 | 0.000 | 8324.5 | 900.9 | 0.984 | False |
| uniform_11 | uniform | 6417.0 | 0.000 | 8324.5 | 0.000 | 1.000 | False |

# 3. Results

## 3.1 Baselines and convergence

Across the synthetic scenarios, the retention baseline produced Lmax=6417.0 kg m-1 and Smax=0.000 kg m-1, whereas the shedding baseline produced Lmax=80.00 kg m-1 and Smax=80.00 kg m-1. These are modeled unit-width masses, not structural loads. The frozen 8% four-metric convergence tolerance was met by 3 of 9 tested resolution combinations. The kinetic-energy proxy was invariant to cell count for the uniform regression case.

## 3.2 Uniform and heterogeneous Pareto sets

At the normalized-distance knee, the selected joint profile had Lmax=3210.6 kg m-1 and Smax=40.00 kg m-1. Relative to the selected uniform knee, these values changed by +3913.3% and -50.0%, respectively. This knee comparison is illustrative and does not establish front-wide superiority.

The complete frontier summary was uniform: 2912 points, normalized hypervolume 0.402, additive epsilon versus uniform 0.000; geometry: 320 points, normalized hypervolume 0.267, additive epsilon versus uniform 0.205; surface: 1731 points, normalized hypervolume 0.402, additive epsilon versus uniform 0.000; joint: 1928 points, normalized hypervolume 0.611, additive epsilon versus uniform 0.000. Because repeated objective pairs can correspond to distinct design profiles, all nondominated design rows were retained in machine-readable results.

## 3.3 Robustness, complexity, and decision scenarios

Under the prespecified perturbation model, joint: median Lmax 85.75 kg m-1 and median Smax 78.10 kg m-1; uniform: median Lmax 5854.6 kg m-1 and median Smax 0.000 kg m-1. These distributions quantify parameter perturbations around selected designs and are not empirical confidence intervals.

Among the prespecified frontier candidates, the robust selection was joint_02 (joint); its configured upper-quantile Lmax and Smax were 103.8 and 103.8 kg m-1. Table 8 compares this result with 2 nominal knees and the remaining candidate set.

Manufacturable segment mapping, generic surface mapping, and slope rounding changed the primary outcomes by the amounts reported in Table 3. Frontier-wide complexity and manufacturing-screen metrics are reported in Table 6; Table 7 reports choices under no, base, and high transition penalties. Labor-scarcity scenarios weight manual intervention directly (Table 5) and were not assigned to actual communities. Table 4 varies snowfall severity, shedding penalties, allowable release, and labor scarcity. Strategy classifications are model-derived labels rather than building-code recommendations.

## 3.4 JMA station-winter scenarios

Daily forcing covered 4 stations and 12 station-winters. Table 9 reports the selected uniform and joint profiles under these observations. Daily aggregation coarsens event timing, so these outputs are supplementary scenario checks and not comparable to hourly shedding observations.

### Table 9. Supplementary JMA daily station-winter evaluation.

| Station | Winter | Days | Design | Lmax | Smax | Events | Mean SSCI |
| --- | --- | --- | --- | --- | --- | --- | --- |
| s47433 | 2021 | 181 | uniform_knee | 345.6 | 345.6 | 92.00 | 0.018 |
| s47433 | 2021 | 181 | joint_knee | 3760.0 | 175.6 | 93.00 | 0.018 |
| s47433 | 2022 | 181 | uniform_knee | 336.0 | 336.0 | 80.00 | 0.018 |
| s47433 | 2022 | 181 | joint_knee | 3903.8 | 168.0 | 82.00 | 0.018 |
| s47433 | 2023 | 182 | uniform_knee | 307.2 | 307.2 | 90.00 | 0.019 |
| s47433 | 2023 | 182 | joint_knee | 3151.5 | 153.6 | 93.00 | 0.020 |
| s47520 | 2021 | 181 | uniform_knee | 316.8 | 316.8 | 63.00 | 0.025 |
| s47520 | 2021 | 181 | joint_knee | 2910.6 | 158.4 | 63.00 | 0.025 |
| s47520 | 2022 | 181 | uniform_knee | 451.2 | 451.2 | 53.00 | 0.038 |
| s47520 | 2022 | 181 | joint_knee | 2282.4 | 225.6 | 52.00 | 0.038 |
| s47520 | 2023 | 182 | uniform_knee | 278.4 | 278.4 | 41.00 | 0.047 |
| s47520 | 2023 | 182 | joint_knee | 563.2 | 139.6 | 41.00 | 0.048 |
| s47575 | 2021 | 181 | uniform_knee | 499.2 | 499.2 | 59.00 | 0.034 |
| s47575 | 2021 | 181 | joint_knee | 2570.7 | 254.8 | 60.00 | 0.034 |
| s47575 | 2022 | 181 | uniform_knee | 278.4 | 278.4 | 53.00 | 0.029 |
| s47575 | 2022 | 181 | joint_knee | 1728.7 | 139.2 | 54.00 | 0.028 |
| s47575 | 2023 | 182 | uniform_knee | 278.4 | 278.4 | 44.00 | 0.036 |
| s47575 | 2023 | 182 | joint_knee | 787.8 | 139.2 | 43.00 | 0.036 |
| s47612 | 2021 | 181 | uniform_knee | 384.0 | 384.0 | 33.00 | 0.052 |
| s47612 | 2021 | 181 | joint_knee | 1140.7 | 192.0 | 33.00 | 0.052 |
| s47612 | 2022 | 181 | uniform_knee | 451.2 | 448.9 | 19.00 | 0.106 |
| s47612 | 2022 | 181 | joint_knee | 749.0 | 224.4 | 19.00 | 0.106 |
| s47612 | 2023 | 182 | uniform_knee | 364.8 | 357.9 | 18.00 | 0.095 |
| s47612 | 2023 | 182 | joint_knee | 441.2 | 178.9 | 18.00 | 0.095 |

# 4. Discussion

The analysis demonstrates how spatial grading can be evaluated against a fully optimized uniform reference rather than a single hand-picked roof. The computed fronts support only conditional statements within the reduced-order model. A larger normalized hypervolume or a favorable selected knee does not imply that every heterogeneous design is better, nor that the same relation persists under unmodeled wind, geometry, or material behavior.

The main design implication is methodological: passive snow-management concepts should be compared as trade-off sets, with manufacturability and operational preferences included explicitly. The generic surface mapping shows how a smooth continuous optimum can be translated into a small material-class vocabulary while measuring the resulting objective loss.

## 4.1 Limitations

The model has not been calibrated or validated against roof-scale shedding measurements. It omits wind redistribution, three-dimensional flow, snow fracture and slab mechanics, cornices, local roof details, heat transfer through the assembly, impact trajectories, drainage blockage, structural response, and pedestrian exposure. Friction, adhesion, compaction, melt, and aging parameters include explicit assumptions. Daily JMA observations do not resolve subdaily events, and ground snowfall or snow depth is not roof snow mass. Accordingly, the study does not establish structural safety, pedestrian safety, code compliance, injury reduction, lifecycle cost, or universal effectiveness.

## 4.2 Future validation

Future work should calibrate interface parameters across temperature, liquid-water content, roughness, aging, and load; validate mass and release timing against instrumented roofs; add wind redistribution and three-dimensional edge effects; and predefine external-validation metrics before fitting.

# 5. Conclusions

A reproducible reduced-order workflow compared complete optimized uniform and spatially heterogeneous passive roof-snow trade-off sets. The results quantify how grading changes modeled retained-mass and discrete-release objectives under synthetic and supplementary JMA forcing, with explicit convergence, ablation, sensitivity, robustness, and complexity analyses. The outputs are hypotheses and design-space evidence for future roof-scale validation, not design approval or safety certification.

# Declarations

Funding: REQUIRED

Declaration of competing interests: REQUIRED

Data and code availability: Source code, immutable public-data snapshots, acquisition metadata, generated result tables, and reproduction instructions are available at https://github.com/bougtoir/functionally-graded-roof-snow-management.

Generative AI statement: During preparation, the author used Devin (Cognition AI) to support coding, public-data acquisition, analysis scripting, and drafting. The author must review and edit the final submission and remains responsible for its content.

CRediT authorship contribution statement: REQUIRED-author confirmation of roles before submission.

# References

1. Giever PM, Sack RL. Similitude considerations for roof snow loads. Cold Regions Science and Technology. 1990. https://doi.org/10.1016/0165-232X(90)90018-R.

2. Thiis TK, Gjessing Y. Large-scale measurements of snowdrifts around flat-roofed and single-pitch-roofed buildings. Cold Regions Science and Technology. 1999. https://doi.org/10.1016/S0165-232X(99)00021-X.

3. Zhou X, Kang L, Yuan X, Gu M. Wind tunnel test of snow redistribution on flat roofs. Cold Regions Science and Technology. 2016. https://doi.org/10.1016/j.coldregions.2016.04.006.

4. Tominaga Y. Computational fluid dynamics simulation of snowdrift around buildings: Past achievements and future perspectives. Cold Regions Science and Technology. 2018. https://doi.org/10.1016/j.coldregions.2017.05.004.

5. Sun X, He R, Wu Y. Numerical simulation of snowdrift on a membrane roof and the mechanical performance under snow loads. Cold Regions Science and Technology. 2018. https://doi.org/10.1016/j.coldregions.2017.09.007.

6. Zhang G, Zhang Q, Fan F, Shen S. Numerical simulations of development of snowdrifts on long-span spherical roofs. Cold Regions Science and Technology. 2021. https://doi.org/10.1016/j.coldregions.2020.103211.

7. Pawluk RE, Rezvanpour M, Chen Y, She Y. A sensitivity analysis on effective parameters for sliding/melting prediction of snow cover on solar photovoltaic panels. Cold Regions Science and Technology. 2021. https://doi.org/10.1016/j.coldregions.2021.103262.

8. Zhang B, Zhang Q, Mo H, Fan F, Lehning M. Considering snow depositions and transport mechanisms in meteorological analysis of predicting snow loads on buildings. Cold Regions Science and Technology. 2022. https://doi.org/10.1016/j.coldregions.2022.103614.

9. Zhou X, Zhang T, Liu Z, Gu M. A study of snow drifting on monoslope roofs during snowfall: Wind tunnel test and numerical simulation. Cold Regions Science and Technology. 2023. https://doi.org/10.1016/j.coldregions.2022.103731.

10. Zou J, Sun X, Zhou H. Sensitivity analysis and practical application of an automatic snow-melting membrane roof. Cold Regions Science and Technology. 2023. https://doi.org/10.1016/j.coldregions.2022.103754.

11. Tominaga Y, Igarashi K, Wakui M, Motoyoshi H, Ito Y. Measurement and estimation of temporal variations of roof snow load on semi-full-scale building model. Cold Regions Science and Technology. 2025. https://doi.org/10.1016/j.coldregions.2025.104445.

12. Fasani D, Cernuschi F, Wolfsperger F, Balordi M, Colombo LPM. Shear and tensile adhesion of dry and wet snow on metallic surfaces. Cold Regions Science and Technology. 2026. https://doi.org/10.1016/j.coldregions.2026.104971.

13. Kimura T. Change of removing technique of snow on roof and recent situation of its technical development. Snow and Ice. 1989. https://doi.org/10.5331/seppyo.51.111.

14. Hokkaido Research Organization, Hokkaido Building Guidance Center. Planning roof-snow treatment for detached houses. Official technical guide. 2017. official guide.

15. Niigata Prefecture. Technical material on characteristics and design ideas for roof-snow treatment. Official prefectural technical material. 2006. official guidance.

16. Takakura M, Ito T, Tomabechi T. Assessment of the effectiveness of snow guards on a sloped roof. AIJ Journal of Technology and Design. 2003. https://doi.org/10.3130/aijt.9.67_1.

17. Tomabechi T, Takakura M, Ito T. Influence of the surface roughness of roofing materials on snow sliding. Journal of Snow Engineering of Japan. 1996. https://doi.org/10.4106/jsse.12.205.

18. Ito T, Tomabechi T, Mitsuhashi H. Evaluation and Maintenance of Snow Sliding Performance of Coated Steel Roofing Used for Sloped Roof. Journal of Snow Engineering of Japan. 1997. https://doi.org/10.4106/jsse.13.244.

19. Asano Y, Iwai K. Experimental Study on the Snow Removal on the Roof by Snow-Melting System in Northern Area of Nagano Prefecture. Journal of Architecture Planning and Environmental Engineering. 1995. https://doi.org/10.3130/aija.60.57_3.

20. Chiba T et al.. Fundamental study for elucidating snow cornice formation mechanism of flat roofs based on wind tunnel test using natural snow. Snow and Ice. 2021. https://doi.org/10.5331/seppyo.83.3_249.

21. Sapporo City. Measures to prevent hazards caused by falling ice and snow. Municipal administrative guidance. 2026. Sapporo Building Standards Ordinance Article 12 guidance.